Sample datasets description

CTD (Conductivity–Temperature–Depth) data

There are two files, a ASCII and a header (metadata) file obtained from a CTD cast at a particular station of a cruise.

File 1: stn298002.HDR

This is a header (metadata) file generated by a Sea-Bird SBE 9 CTD system using the Seasave software.

Key information:

- Ship: FORV Sagar Sampada

- Cruise: CR 297

- Station: STN298002

- Latitude: 20° 46.152′ N

- Longitude: 65° 13.566′ E

- Bottom Depth: 2659 m

- System: Sea-Bird SBE 9

- Software: Seasave Win32 v5.31

| Index | Variable | Description | Unit |
| --- | --- | --- | --- |
| 0 | prDM | Pressure, Digiquartz | dbar |
| 1 | c0mS/cm | Conductivity | mS/cm |
| 2 | sbeox0ML/L | Dissolved Oxygen (SBE 43 sensor) | ml/l |
| 3 | t090C | Temperature (ITS-90 scale) | °C |
| 4 | xmiss | Beam Transmission (Chelsea/Seatech/Wetlab C-Star) | % |
| 5 | bat | Beam Attenuation (C-Star) | 1/m |
| 6 | depSM | Depth (salt water) | m |
| 7 | sal00 | Salinity | PSU |
| 8 | potemp090C | Potential Temperature (ITS-90) | °C |
| 9 | sigma-t00 | Sigma-T (density anomaly) | kg/m³ (implied) |

File 2: stn298002.asc

This is the main data file in ASCII text format, corresponding to the above header.

Structure:

The first line lists column headers (matching the variable names in the header file). Each subsequent line contains numerical data for a single depth or scan record. Values are space-delimited and recorded at roughly 1 dbar (≈ 1 m) depth intervals.

Automated Weather Station (AWS) data

The primary use of this text file, met.txt, is to serve as a raw data log for scientific, navigational, and operational analysis related to a cruise. The attached file, met.txt, is a plain text (ASCII) data log from an Automated Weather Station (AWS) on a cruise. Its format is characterised by time-stamped, space-separated records that combine navigation and environmental data into a single line, grouped by specific record tags.

Structure and Delimiters

- File Type: Plain text log file.

- Record Structure: Each line represents a single data record and is divided into three distinct blocks: $GPS, $MET, and $SST.

- Field Delimiter: Data fields within each record are primarily separated by spaces.

- Header Row: The file begins with a header row defining the parameters in each block.

| Block | Tag | Parameters Included |
| --- | --- | --- |
| Navigation | $GPS | Date, Time, Latitude (and Hemisphere), Longitude (and Meridian), Vessel Speed, Course, and Heading. |
| Meteorological | $MET | Batt (Battery voltage), Ref, Wmax (Maximum Wind Speed), True_wind (True Wind Speed/Direction), Raw_wind (Raw Wind Speed/Direction), ATemp (Air Temperature), Humi (Relative Humidity), Pressure, Dew (Dew Point), SR (Solar Radiation), and RF (Rainfall). |
| Sea Surface | $SST | STemp (Sea Surface Temperature), Sal (Salinity), Pu (likely a placeholder or instrument status), and depth. |

Acoustic Doppler Current Profiler (ADCP)

The attached file, ADCP.txt, is a processed data log from an Acoustic Doppler Current Profiler (ADCP) instrument. This instrument is specifically used on a cruise to measure water current velocities across different depth layers of the ocean. The file is a comprehensive record of the cruise track and the corresponding vertical profile of ocean currents,

| Field | Description | Unit |
| --- | --- | --- |
| Ens | Ensemble Number (Sequential measurement ID). | - |
| YR, MO, DA, HH, MM, SS, HH | Timestamp (Year, Month, Day, Hour, Minute, Second, Hundredth of Second). | - |
| Hea | Vessel Heading (Direction the vessel is pointed). | Degrees (deg). |
| FLat, FLon | Latitude and Longitude at the First ping of the ensemble. | Degrees (deg). |
| LLat, LLon | Latitude and Longitude at the Last ping of the ensemble. | Degrees (deg). |
| BD1, BD2, BD3, BD4 | Bottom Depth (Depth to the seabed measured by each of the four ADCP beams). | Centimeters (cm). |

The core of the file consists of current velocities measured in up to 20 depth bins.

- "Eas" (Eastward Velocity): The component of the water current moving East (positive) or West (negative). Measured in millimeters per second (mm/s) for 20 bins.

- "Nor" (Northward Velocity): The component of the water current moving North (positive) or South (negative). Measured in millimeters per second (mm/s) for 20 bins.

- "Mag" (Magnitude): The speed of the water current (calculated from the Eas and Nor components). Measured in millimeters per second (mm/s) for 20 bins.

- "Dir" (Direction): The direction the water current is flowing toward. Measured in degrees (deg) for 20 bins.

Molecular datasets

Sample datasets of DNA/Nucleotide in gene formats can be accessed from https://www.ncbi.nlm.nih.gov/nuccore/?term=

Sample datasets of DNA/Nucleotide in raw data formats can be accessed from https://www.ncbi.nlm.nih.gov/sra/

Barcode data of all organisms can be found at https://boldsystems.org/